DPTI REQUIREMENTS FOR THE MEASUREMENT OF EARTHWORK VOLUMES – EST – 03

(ESTIMATE DRAWING)

An  Estimate Drawing is required to assist estimators in accurately determining earthwork volumes.

This Estimate Drawing shall be in .DWG or .DXF format, be presented using the standard DPTI Sketch Titleblock and include the following entities:

- Contours depicting the existing surface

- Contours depicting the proposed surface

- Boundaries around the extent of each individual pavement type

Contours:

- Shall be created at a suitable interval that will accurately depict the relevant surface without causing the drawing to become too cluttered.

- Shall extend across the entire surface being modelled (for example, the existing surface should not be ‘trimmed out’ within the extents of the design)

- Shall be continuous unbroken polylines

- Shall have an elevation equivalent to their level.

- Shall not include labels indicating their level.

- Shall conform to DP001 in terms of layers and colour

Pavement Boundaries:

- Shall be closed polylines that represent the extent of each individual pavement type.

- Each pavement type is to be defined with a separate boundary.  There may be multiple pavement boundaries for one pavement type if they are physically separated

- Shall conform to DP001 in terms of layers and colour

CAD Support File resources:

- Estimate Drawing.inp 			MX input file for producing an Estimate Drawing

- DPTI-RD-SketchTitleBlock.dwg	Standard DPTI Sketch titleblock.